# PCAN & TPMS Project Detailed Report

# 1. Executive Summary

This report details the operational logic, data flow, and architecture of the Vehicle Health Monitoring System. The system integrates hardware interfacing (PCAN-USB), wireless sensor protocols (BLE), and real-time visualization (React Frontend) to provide a complete diagnostic dashboard.

# 2. System Architecture

The application is built on a decoupled Client-Server architecture.

## 2.1. Backend Layer (Python/FastAPI)

Core Responsibility: Hardware abstraction and API serving.

Key Components:

FastAPI Server: Asynchronous web server handling HTTP requests and WebSockets.

PCAN Service: A singleton service managing the dedicated thread for reading CAN bus messages via the PCANBasic API.

BLE Automation Module: A task-based system that manages long-running Bluetooth Low Energy connection sequences and testing.

### 2.2. Frontend Layer (React/Vite)

Core Responsibility: User interaction, state management, and real-time rendering.

Key Components:

Dashboard Page: Renders the 3D Truck model and charts using Chart.js.

CAN Console: Provides a low-level debugging interface for raw hex messages.

BLE Test Control: Interface for configuring and running automated test suites.

# 3. Operational Data Flows

## 3.1. System Startup & Initialization

1. The Backend starts and initializes the `PCANService` and `TPMSService` singletons.
2. The Frontend loads and checks the API health status (`/api/pcan/status`).
3. If PCAN hardware is detected, the Frontend automatically requests initialization with default settings (500k baud).
4. A WebSocket connection is established at `ws://<host>/ws` to listen for global broadcast events (e.g., BLE logs).

## 3.2. CAN Bus Data Pipeline

This flow describes how data moves from the vehicle bus to the user screen:

Step 1: Hardware Interrupt

The PCAN-USB driver receives a CAN frame from the physical bus.

Step 2: Service Polling

The `PCANService` background thread runs a continuous loop (microsleep 0.05s) calling `Read()`. Received messages are pushed into a thread-safe `deque` buffer.

Step 3: Frontend Fetch

The React Frontend polls the `/api/pcan/read` endpoint every 50ms.

Step 4: Visualization

New messages are processed by `TPMSDashboard.jsx`. If the message ID matches a configured Tire ID, the payload is parsed for Pressure/Temperature/Battery and the state is updated.

## 3.3. BLE Automation Flow

1. User selects a target device and test configuration (UUIDs, retries) in the UI.
2. A POST request to `/start-test` initiates an asynchronous task in `main.py`.
3. The `BLETestAutomation` class connects to the device using `BleakClient`.
4. Commands are executed in chunks. For each command:
    a. Command is written to the WRITE characteristic.
    b. System waits for a Notification response.
    c. Response is validated (Pass/Fail) and logged.
5. Every log entry is broadcast immediately via WebSocket to the frontend log window.

# 4. Detailed Logic Specifications

## 4.1. Tire Status Classification

The system visually indicates tire health using color codes derived from these specific thresholds:

| Metric | Critical (Red) Action Required | Warning (Orange) Caution | Normal (Green) Good |
| --- | --- | --- | --- |
| Pressure (PSI) | < 20 OR > 120 | < 30 OR > 100 | 30 - 100 |
| Temperature (°C) | > 80 | > 60 | < 60 |
| Battery (Voltage) | < 2.5 V | < 3.0 V | > 3.0 V |

Note: A "Critical" status overrides "Warning", and "Warning" overrides "Normal".

## 4.2. Command Validation Logic

Responses from BLE devices are automatically graded:

PASS (P): Returned value is exactly "0".

FAIL (F): Returned value is negative (e.g., "-1").

UNKNOWN: Any other value or malformed string.

# 5. File Structure & Purpose

A high-level guide to the project verification files:

| File Path | Description of Responsibility |
| --- | --- |
| backend/app/main.py | Server entry point. Orchestrates the startup sequence and routes requests. |
| backend/app/services/pcan_service.py | Hardware Driver Layer. Contains the infinite loop thread for CAN reading. |
| backend/app/routers/*.py | API Interface. Defines the HTTP endpoints exposed to the frontend. |
| frontend/src/pages/TPMSDashboard.jsx | Primary UI Logic. Handles 3D rendering, chart data smoothing, and state updates. |
| frontend/src/services/api.js | Network Layer. Centralized configuration for all HTTP/WebSocket calls. |